Norwich Technical High School - Electronics Technology		Date Updated:

7 Mahan Drive

Norwich, CT 06360

TITLE: Heat Press SOP

APPROVALS:

Author: Daniel Larose, Sam Moriarty and Dimitri Torres	Date:	1/3/2024

Review:		Date:

- Revision History:

| INITIALS | REV | DATE | SUMMARY OF CHANGES |
| --- | --- | --- | --- |
| DL / SM / DT | A | 1/3/2024 | Original |

- Purpose:

  - To transfer blue sheet ink to the copper boards.

- Safety: While the heat press is on don’t touch it, as it gets extremely hot.

  - It also holds on to heat very well so you might need to let it cool down before you grab your board.

- Materials:

- Copper board

- Printed Bluesheet

- Heat Press

- Procedure:

| Step | Procedure | Picture |
| --- | --- | --- |
|  | Acquire a printed bluesheet and cut out your circuit. Acquire a copper board and cut it out to a size that is slightly bigger than your bluesheet, if possible, find a board that doesn't need to be cut. |  |
|  | Place the circuit on a copper board shiny side up. |  |
|  | Pre-heat the heat press to 325 degrees fahrenheit. The pressure is already set, no need for adjustment. |  |
|  | Place the board and sheet on the heat press with the bluesheet facing up. Be careful not to touch the inside of the heat press while doing so. |  |
|  | Press the board for about 3 minutes. |  |
|  | Carefully remove the board from the heat press, ideally with a putty knife or something similar, if one is not available, use your nail to drag it off. Beware that the copper will be very hot. Let the board sit for a few minutes to cool. (Please note images from this point on are acted out as best as possible, but the board was not actually pressed) |  |
|  | Check the board, if the paper is still peeling from the copper, put it back on the heat press for another 30 seconds. Repeat from step 6 as necessary until the bluesheet sticks to the copper board. |  |
|  | Rinse board in clean, cold water. |  |
|  | Remove and dispose of the excess film from the board. And then proceed to the acid, see SOP-001 for directions on working with acid. |  |