Norwich Technical High School - Electronics Technology		Date Updated: 2/28/2024

7 Mahan Drive

Norwich, CT 06360

TITLE:	Fusion to CNC software - Standard Operating Procedure (SOP)

APPROVALS:

Author:	N. Willbanks	Date:	1/7/2026

Review:		Date:

- Revision History (Create Columns as Needed):

| NAME | REV | DATE | SUMMARY OF CHANGES |
| --- | --- | --- | --- |
| N. Willbanks | A | 1/7/2026 | Initial Release |

Purpose:

- To produce a Gerber & drill file on Fusion Electronics that our shop CNC can use to cut out the circuit traces on our CNC machine

- See SOP-### for details on cutting the circuit on our shop CNC

Safety:

- Eye protection during circuit cutting

- Materials:

- Software:

  - Fusion360

  - CopperCAM

  - UGS (Universal Gcode Sender)

- CNC Cutter

- Appropriately sized drill bits

- Copper sheet(s)

- Flashdrive

Procedure:

| Step | Procedure | Picture |
| --- | --- | --- |
|  | Create your circuit on Fusion Electronics |  |
|  | From the PCB Design View, Select the “Manufacturing” tab |  |
|  | Select the gear icon to confirm the output settings match what is required for a successful output |  |
|  | By default, output settings should look like this |  |
|  | Select the first arrow icon next to the gear icon to export your Gerber, Drill, Assembly, & Drawing files |  |
|  | Give that file a name & save it to your flashdrive (or other location where you can easily find it) |  |
|  | When you unzip that file, “DrillFilles” & “GerberFiles” contain the files you need to cut your circuit on the CNC |  |
|  | Note: The Drill file extension type might need to be changed from “.xln” to “.exl” unless you adjusted that output setting already in Fusion |  |
|  | Open CopperCAM |  |
|  | In CopperCAM, open a new circuit |  |
|  | Select your Gerber File (usually “Copper_Top” or “Copper_Bottom”) from “GerberFiles” for your “Solder side.” Select your exl file from “DrillFiles” for “Drills.” “X” out to remove the “Component side” & “Cut-out.” You don’t need files for either of those. | (Note: You usually want to uncheck “Call milling process immediately”) |
|  | Your file should look something like this when you are done, & can be modified further from within CopperCAM |  |